Flash Tool Download Guide

Step1. Power off the phone

Step2. Connect the phone and Computer with GSM cable

Step3. Double click

Step4. Set “Baud rate”=921600

Step5. Set “COM port”=COMx

You must select a correct COM , the “x” maybe is 1,2,3…

Step6. Set “Format Begin Address and Length”=Auto Format FAT

Step7.Click [Download Agent] , and select “MTK_AllInOne_DA.bin”

Setp8. Click [Scatter-loading] , and select “scatGC09.txt”

Step9. Click [ROM] , and select Code

The code format as “Code_GC09_2.05_38B8_[17A0].bin”

Step10.Click [Download]

Step11. Press power key on the phone at the same time, when the Flash tool begin to download the GSM code ,you can release it.

Step12. When you see this information , it means the downloading has finished

Step13. Reset the phone and you can use it now.